Electric Fruit Crusher ESE-018

Original Operating Manual (EN)

Dear Customer!

This Manual will help you learn how to use the crusher effectively.

Upon purchase, check the completeness indicated in Table 1, the absence of external and mechanical damage.

ATTENTION!

Please read this Operating Manual carefully before using the crusher and save it for future reference. Loss of this Manual does not relieve you of the responsibility to comply with it.

Due to the continuous improvement of the products, the manufacturer reserves the right to make changes to the design and appearance of the crusher, without affecting its technical characteristics. In this regard, the contents of the Operating Manual may not fully correspond to the purchased crusher. Keep this in mind when reading this Manual. The appearance of the crusher may differ from the images in the figures of this Manual.

The crusher is allowed to be transferred for temporary use or rental only to those people who are familiar with this model, have trained to handle it and have read this instruction, in which case the Operating Manual must be attached.

1. General instructions

1.1. The electric fruit crusher ESE-018 (hereinafter referred to as the Crusher) is intended for crushing apples, pears, grapes and other fruits and berries, as well as vegetables (except stone fruits, such as plums, apricots, etc.) in private households and small farms.

1.2. The Crusher is not intended for use in environments such as canteens and kitchens for staff or visitors in stores, offices, restaurants, snack bars, hotels, guest houses, cruise ships, airplanes, etc.

1.3. Using the Crusher for any other purposes not provided for in this Manual is a violation of the conditions of warranty service and terminates the manufacturer's warranty.

1.4. The manufacturer is not responsible and is not obliged to make a free repair of the product, return the cost of the product and/or indemnify for damage or defects resulting from the use of the Crusher for other purposes, in violation of this instruction or state regulations in the country of use, as well as in the event specified in paragraph 11.4 of this Manual.

1.5. The Crusher is designed to work indoors or under a canopy at ambient temperatures from 0ºС to +40ºС.

1.6. During operation and storage, prevent dropping and impacts.

2. Complete set and structure of the electric crusher ESE-018

Table 1

Complete set of the electric crusher ESE-018

| Item | Quantity (pcs.) |
| --- | --- |
| Crusher | 1 |

Crusher structure

1 - Top cover of the crusher casing

2 - Bottom part of the crusher casing

3 - Crushing disc

4 - Motor

5 - Motor support plate

6 - On/Off Switch

7 - Power cable

Installation of the crushing disc

1 - Fixing bolt

2 - Round flat washer

3 - Figured flat washer

4 - Crushing disc

5 - “Glass”

6 - Adapter bushing

3. Technical characteristics of the electric crusher ESE-018

| Motor power: | 0,22 kW |
| --- | --- |
| Productivity: | 200-300 kg/h |
| Diameter of the feed opening: | 85 mm |
| Dimensions: | 23х34х43 cm |
| Weight: | 8,5 kg |
| Power voltage: | 120V, 60 Hz |
| Motor rotation frequency: | 1300 rpm |
| Casing and crushing disc material: | Food-grade stainless steel |

4. General safety rules

4.1. Read this Manual carefully. Familiarize yourself with the Crusher before operating it. Observe safety precautions when operating the product. Violation of this Operating Manual and safety rules can lead to accidents.

4.2. Only one person can use the Crusher at a time. Bystanders must be at a safe distance. Children and especially animals must be kept away from the place of use of the Crusher.

4.3. Do not use the Crusher if you are tired, under the influence of medicines, drugs or alcohol.

4.4. Use the Crusher only in the daytime or in good artificial light.

5. Safety instructions

ATTENTION!

The Crusher must be connected to a single-phase mains with a residual current circuit breaker for leakage current more than (≥) 30 mA, overload and short circuit protection (circuit breaker for 6-10A) and protective earth.

5.1. Only plug into a power outlet with a protective earth.

5.2. It is necessarily to fix the Crusher before starting work, in order to avoid its overturning during operation.

5.3. You must turn on the device into the mains only after you have checked all the fastenings and the reliability of installation of all structural elements.

5.4. Make sure that hair, clothing and gloves do not come in contact with the rotating parts of the Crusher.

5.5. Use only as directed, i.e. for crushing fruits, berries and vegetables (except stone fruits, such as plums, apricots, etc.). Do not use to crush frozen fruits, berries and vegetables, as well as ice cubes.

5.6. If the crushing disk is blocked, immediately turn off the Crusher and disconnect the plug from the outlet. Clean it from jammed fruits, berries and vegetables and continue to work.

5.7. Monitor the condition of the power cable and use the Crusher only if the cable is not damaged. If the power cable is damaged, it must be replaced by the manufacturer, its service agent or other qualified person.

5.8. If a malfunction or damage is detected that could jeopardize the safety of operating or the operation of the device, and which cannot be repaired by the user, it is forbidden to use the Crusher.

5.9. The Crusher can be used independently only by physically and mentally healthy individuals without impairing sensory abilities over the age of 18 years, who are trained in its operation skills and familiar with these Operating Manual, which must be stored in a place accessible to the user.

5.10. Keep the Crusher out of the reach of children.

5.11. When working with the Crusher, carefully monitor the safety of persons in the working area of the device.

5.12. Be careful when removing the crushing disc; there is a chance of injury from the blades.

5.13. Disassemble and flush the Crusher after rotating parts have completely stopped and it’s disconnected from the power supply.

5.14. Provide regular maintenance and cleaning of the device.

5.15. In case of motor jamming, turn off the Crusher using the switch. Disconnect the plug from the outlet. Disassemble and clean the Crusher from fruits, berries and vegetables jamming the crushing disk. If motor jamming occurred for another reason, then contact the authorized service centre or other qualified persons to repair the crusher.

5.16. In case of the Crusher motor short-circuit failure, disconnect the Crusher by disconnecting the plug from the outlet and contact an authorized service centre or other qualified persons.

5.17. In case of water splashing onto the Crusher, turn off the Crusher using the switch and disconnect the plug from the outlet. Wipe the Crusher with a rag and dry it. Contact an authorized service centre or other qualified persons.

5.18. In case of mechanical instability (increased vibration, etc.), turn off the Crusher using the switch. Disconnect the plug from the outlet. Check the tightness of all bolts. If the malfunction persists, contact an authorized service centre or other qualified persons.

5.19. If the Crusher is not used for any reason, it must be disconnected from the mains by disconnecting the plug from the outlet.

6. IT IS PROHIBITED!

6.1. To switch on the Crusher without the top cover of the crusher casing in place.

6.2. To touch the rotating parts with your hands while the Crusher is in operation.

6.3. To insert any tools or objects, except material intended for crushing, into the feed opening during operation of the Crusher.

6.4. To put your hands in the feed opening while the Crusher is working.

6.5. To use the device with a damaged power cable.

6.6. To use the Crusher for children under the age of 18, as well as for people with limited physical, sensory or mental abilities.

6.7. To use the Crusher in rain or snow in an open area.

6.8. To pour fruits, berries and vegetables into the feed opening from a height of more than 0.5 m from the crushing disk.

6.9. To clean the device casing with a jet of water or using chemical agents. Under the tap, only the removable crushing disc and top cover can be washed. The device casing may be cleaned with a damp cloth or sponge to prevent moisture from entering the device.

6.10. To repair the Crusher yourself or attempt to make changes to its design. Repair should be carried out in a specialized service centre.

6.11. To immerse the Crusher casing, including the crusher motor, in water or place them under a stream of water.

7. Preparing for work

7.1. Remove the product from the packaging. Before using the Crusher for the first time, thoroughly rinse all parts that come into contact with the juice with a sponge and detergent. Make sure that water and snow do not fall on the crusher motor.

7.2. Place the product on a flat surface, for example, on the edge of the table, and place a container for collecting crushed fruits, berries and vegetables under the crusher outlet.

7.3. Fix the Crusher on the table surface, for example, using a screw clamp or 2 screws (there are two holes for fixing screws/self-tapping screws on the base plate from the rear edge).

8. Working with the Crusher

8.1. Wash fruits, berries and vegetables if they were collected from the ground (since particles of ground on the fruits can dull the crushing disc and also spoil the taste of the juice).

8.2. Insert the plug into a power outlet with a protective earth and turn on the Crusher using the switch on the motor housing.

8.3. Start crushing fruits, berries and vegetables by pouring them into the feed hopper. Monitor the capacity of the container with crushed fruits and empty it in time, otherwise the fruits will have nowhere to get out and the crushing disk may become blocked.

8.4. After working with the Crusher, turn off the Crusher using the switch and disconnect the plug from the outlet.

8.5. After using the Crusher, remove the top cover of the crusher casing by unscrewing 3 wing nuts, remove the crushing disc by unscrewing the fixing bolt with a wrench, remove the remaining crushed raw materials and rinse both parts of the crusher casing and the crushing disc with water, while making sure that water does not get onto the motor. The device casing may be cleaned with a damp cloth or sponge to prevent moisture from entering the device. After washing the Crusher, wipe all parts dry.

Watch the video of work with the Crusher on our website or on our YouTube channel:  https://www.youtube.com/channel/UCKFdbuOASzI3aWwq2rgMoBA/videos

9. Maintenance

9.1. At the end of each working day, it is necessary to clean the Crusher (Paragraph 8.5).

9.2. If the crushing disc is dull, then it can be sharpened with a round file or a conical burr by contacting an authorized service centre or other qualified persons.

9.3. Monitor the condition of the motor and power cable. If necessary, contact an authorized service centre or other qualified persons.

9.4. When servicing, replacing parts, cleaning, etc. the product must be disconnected from the mains by disconnecting the plug from the outlet.

10. Storage

Store in a dry, well-ventilated area, eliminating atmospheric precipitation, direct sunlight and building dust.

The presence in the air of acid fumes, alkalis and other aggressive impurities is not allowed.

The Crusher must not be accessible to children during storage.

11. Manufacturer's warranties

11.1. The manufacturer warrants the correct operation of the product, subject to the storage, installation and operation rules specified in this Operating Manual, as well as the requirements of state regulations in the country of use of the product.

11.2. The warranty period for the product is set by the manufacturer and is 36 months from the date of sale of the product to the consumer. In the absence of a sale date, the warranty period is calculated from the date of manufacture of the product.

11.3. The manufacturer undertakes to make a free repair during the warranty period if the following conditions are met:

- The operation of the product was carried out in strict accordance with this Operating Manual. Otherwise the repair is carried out at the expense of the buyer;

- There is a positive decision by experts of the manufacturer’s quality department to carry out warranty repairs of the product;

- The product arrived in its clean form, without mechanical damage, with a complete set;

- There is a consumer statement indicating the reasons for the failure of the product, photographs or video materials;

- There is a technically sound report from a trading organization;

- There is a receipt, bank payment order or other document confirming the purchase of the product.

11.4. Manufacturer's warranties do not apply to the following cases:

- If a defect occurs as a result of deliberate and/or erroneous actions of the consumer when using the product;

- In the event of breakage or damage to the product due to natural disaster, illegal actions of third parties;

- In the event of damage caused by ingress of foreign substances, objects, liquids;

- If the malfunction is caused by direct or indirect action of mechanical forces, chemical, thermal or physical effects;

- In the event of disassembly of the product, design changes or other interventions not provided for in this Operating Manual.

SIA Apple Press Ltd does not bear any responsibility and is not obliged to indemnify either moral or material damage if any damage is caused to the user or other individuals, including children and animals, when using the Crusher, including their health (any bodily injuries or psychological injuries) due to non-compliance with the conditions of this Operating Manual and in the absence of manufacturer's fault. Upon the occurrence of the above circumstances, all responsibility for damages rests with the victim or the guilty person. SIA Apple Press Ltd does not bear any responsibility for any negative consequences that occurred when using the product, violating the requirements of this Operating Manual, including damage to the Crusher caused by non-authorised introduction of any changes to the design of this product.

Manufacturer:

SIA «Apple Press Ltd»

Registration number: 40203130458

VAT number: LV40203130458

Address: Maskavas iela 260K-1-13, Riga, LV-1063, Latvia